Q1. What is Metrology?

Metrology is the science of measurement. It includes all theoretical and practical aspects of measurement, regardless of the measurement uncertainty and the field of application. In engineering, it specifically focuses on the standards, units, and instruments used to ensure precision in manufacturing.

Q2. What is allowance?

Allowance is the intentional difference between the maximum material limits of two mating parts (the hole and the shaft). It is used to determine the type of fit.

- Positive Allowance: Results in a clearance fit.

- Negative Allowance: Results in an interference fit.

Q3. What is meant by clearance?

Clearance is the difference between the size of the hole and the size of the shaft when the hole is larger than the shaft. This creates a gap that allows for relative movement (like rotation or sliding) between the parts.

Q4. Define interference.

Interference occurs when the size of the shaft is larger than the size of the hole. In this case, the parts must be forced together (often using heat or pressure), resulting in a permanent or semi-permanent joint with no relative movement.

Q5. What is a transition fit?

A transition fit is a compromise between a clearance and interference fit. Depending on the actual sizes of the manufactured parts within their tolerance zones, the resulting fit may be either a small clearance or a small interference. It is typically used for applications requiring accurate positioning.

Q6. What are the types of tolerances?

Tolerance is the total permissible variation in a dimension. The two primary types are:

- Unilateral Tolerance: The variation is permitted in only one direction from the nominal size (e.g.,

).

- Bilateral Tolerance: The variation is permitted in both directions from the nominal size (e.g.,

).

Q7. What is the least count of vernier calipers? Show calculation of least count.

The Least Count (LC) is the smallest distance that can be accurately measured by the instrument.

Calculation:
The formula for Least Count is:

Standard Example:

- Assume

.

- If

main scale divisions are divided into

vernier scale divisions:

Q8. What is Line standard and End standard?

- Line Standard: The measurement is taken between two engraved lines (e.g., a steel rule or a yardstick).

- End Standard: The measurement is taken between two flat, parallel surfaces (e.g., slip gauges or a micrometer's anvils).

Q9. What is a feeler gauge used for?

A feeler gauge consists of a series of thin steel blades of different thicknesses. It is used to measure gap widths or the clearance between two mating parts, such as spark plug gaps or valve clearances in engines.

Q10. What is Slip gauge? Explain the phenomenon ‘Wringing’.

Slip Gauges (also known as Gauge Blocks) are precision-ground steel or ceramic blocks used as a standard for linear measurement.

Wringing:
Wringing is the process of sliding and pressing two slip gauges together so that they adhere to each other with a high degree of force. This happens due to molecular attraction and air pressure, allowing multiple gauges to be stacked to create a specific, highly accurate dimension.